Conceptual Design for an Autonomous Orbital Vehicle for

Space Debris Collection and Decommissioning of Defunct Satellites

Adithya Boyini

High School Student(Independent Research)

Last updated: Jan 15, 2026

—-------------------------------—-----------------------------—-----------------------------—-----------------

Abstract

Space debris poses an escalating threat to operational satellites, crewed missions, and the long-term sustainability of space activities. With millions of defunct satellites, spent rocket stages, and collision fragments orbiting Earth at velocities exceeding 7 km/s, the risk of catastrophic collisions grows exponentially. Current debris mitigation strategies focus on prevention, while active removal concepts remain costly, single-target missions unsuitable for addressing the scale of the problem.

This research presents a conceptual design for the ADCS-1 (Autonomous Debris Collection Spacecraft), an innovative orbital vehicle capable of autonomously identifying, tracking, capturing, and deorbiting multiple space debris objects in a single mission. The spacecraft employs a hybrid capture system combining deployable nets and a 7-degree-of-freedom robotic arm, providing versatility to handle diverse debris types ranging from 10 cm to 2 meters in size. Electric propulsion via Hall-effect thrusters enables efficient orbital maneuvering with a total ΔV budget of 1,500 m/s, supporting missions lasting 30-90 days.

The design integrates proven space technologies including LIDAR-based tracking, AI-driven autonomous navigation, and solar-electric power generation. Detailed orbital mechanics analysis demonstrates the technical feasibility of multi-target rendezvous sequences using Hohmann transfers and optimized propellant consumption. Economic analysis shows the concept could reduce per-debris removal costs from $100-300 million to $3-5 million through multi-target capability, representing a 10-30× improvement over existing single-target approaches.

Key innovations include:

(1) autonomous AI algorithms enabling extended operations with minimal ground control.

(2) adaptive capture mechanisms suited to both tumbling fragments and stabilized defunct satellites.

(3) scalable design permitting fleet operations to stabilize debris population growth.

(4) modular architecture compatible with commercial launch vehicles at a 1,730 kg launch mass.

The research acknowledges critical limitations including orbital plane change constraints, debris characterization uncertainties, and the need for extensive simulation validation before hardware development. A comprehensive roadmap for future work outlines required simulations (orbital dynamics, capture mechanics, AI validation), hardware prototyping, and a phased demonstration mission approach spanning 7-10 years to operational capability.

This conceptual design establishes that active debris removal is technically feasible with current technologies and economically viable at scale. With appropriate investment and international cooperation, multi-target autonomous debris collection spacecraft could begin reversing decades of orbital pollution within this decade, ensuring sustainable access to space for future generations.

—-------------------------------—-----------------------------—-----------------------------—-----------------

1.Introduction

1.1 Background: The Space Debris Crisis:-

Since the launch of Sputnik 1 in 1957, humanity has sent thousands of satellites, rockets, and spacecraft into Earth's orbit. However, not all of these objects remain functional. Over decades of space activity, Earth's orbital environment has become increasingly crowded with debris—ranging from defunct satellites and spent rocket stages to fragments from collisions and explosions.

What is space debris?

Space debris, also called orbital debris or "space junk," consists of non-functional human- made objects orbiting Earth. This includes:

- Inactive satellites that have reached end-of-life

- Fragments from satellite collisions (such as the 2009 Iridium-Cosmos collision)

- Remnants from anti-satellite weapon tests

- Paint flecks, bolts, and other small particles

- Spent rocket stages and mission-related debris

According to the European Space Agency (ESA), there are approximately:

- 36,500+ objects larger than 10 cm

- 1,000,000+ objects between 1-10 cm

- 130,000,000+ objects smaller than 1 cm

Why is space debris dangerous?

The danger of space debris stems from the extreme velocities at which objects travel in orbit. In Low Earth Orbit (LEO), debris travels at approximately 7-8 km/s (15,000-17,500 mph). At these speeds, even a small paint chip can cause significant damage to operational satellites and crewed spacecraft.

Key risks include:

1. Collision Cascades (Kessler Syndrome): A theoretical scenario where the density of

objects in orbit becomes high enough that collisions generate more debris, triggering a

cascade effect that could render certain orbital regions unusable for generations.

2. Threat to Active Missions**: The International Space Station regularly performs avoidance maneuvers to dodge debris. Satellites worth millions of dollars face constant collision risk.

3. Launch Window Constraints : Debris fields limit safe launch trajectories and timing.

4. Economic Impact : Each collision can destroy assets worth hundreds of millions of

dollars and create thousands of new debris fragments.

Current Debris Mitigation Limitations:

Existing approaches to space debris management fall into two categories: prevention and removal.

Prevention measures include:

- Post-mission disposal guidelines (25-year deorbit rule)

- Passivation of spent rocket stages

- Design for demise (ensuring objects burn up on reentry)

Active removal concepts remain largely theoretical or in early testing:

- ESA's ClearSpace-1 mission (planned)

- JAXA's tether experiments

- RemoveDEBRIS mission demonstration

However, these solutions face significant limitations:

- High cost per debris object removed

- Limited to large, trackable objects

- Require individual missions for each target

- No autonomous, scalable solution exists

1.2 Problem Statement:-

Currently, there is no cost-effective, autonomous system capable of tracking, capturing, and deorbiting multiple pieces of space debris in a single mission. Existing concepts require expensive, single-target missions and lack the scalability needed to address the growing debris population. The space industry urgently needs an innovative solution that can:

- Operate autonomously without constant ground control

- Capture multiple debris objects per mission

- Function across various debris sizes and types

- Utilize efficient propulsion for orbital maneuvering

- Provide safe, controlled deorbiting mechanisms

1.3 Objective of This Research:-

The objective of this research is to develop a conceptual design for an autonomous orbital vehicle capable of identifying, tracking, capturing, and safely deorbiting space debris and defunct satellites.

Specific goals include:

1. Design a spacecraft architecture optimized for debris collection

2. Develop a capture mechanism suitable for various debris types

3. Calculate orbital mechanics requirements (ΔV budget, transfer orbits)

4. Propose an autonomous navigation and tracking system

5. Analyze the technical feasibility and limitations of the concept

6. Establish a framework for future simulation and prototype development

This conceptual design aims to provide a foundation for a scalable, practical solution to one of the most pressing challenges facing modern space operations.

—-------------------------------—-----------------------------—-----------------------------—-----------------

2. Literature Review

2.1 Overview of Existing Debris Removal Concepts:

The challenge of active debris removal has attracted significant research and development efforts from space agencies and private companies worldwide. This section reviews the major approaches that have been proposed or demonstrated.

2.2 Net-Based Capture Systems:

Net-based capture has been explored by several missions, most notably the RemoveDEBRIS mission conducted by the Surrey Space Centre in 2018. This mission successfully demonstrated the deployment of a net to capture a simulated debris target in orbit.

Advantages:

- Can capture irregularly shaped objects

- Flexible approach minimizes damage to target

- Proven technology in orbital environment

Limitations:

- Limited range (typically <10 meters)

- Requires precise targeting

- Net deployment is irreversible

- Difficulty with very large objects

2.3 Robotic Arm Systems:

Robotic manipulator systems have heritage from decades of spacecraft servicing operations. The Canadarm on the Space Shuttle and Canadarm2 on the ISS demonstrate the maturity of this technology. The e.Deorbit mission concept by ESA proposed using a robotic arm for debris capture.

Advantages:

- High precision and control

- Can reposition debris safely

- Proven reliability in space operations

- Allows for inspection before capture

Limitations:

- Requires close proximity operations

- Target must be cooperative or predictable

- Arm mechanisms are complex and massive

- Energy-intensive for large debris

2.4 Harpoon Systems:

The RemoveDEBRIS mission also tested a harpoon system, successfully penetrating a target panel in 2019. This method involves firing a projectile attached to a tether into the debris object.

Advantages:

- Can engage from moderate distances

- Effective for solid, metallic targets

- Provides secure attachment

Limitations:

- Causes impact damage and may create secondary debris

- Ineffective against hollow or fragile structures

- Single-use per harpoon

- Difficult retrieval if harpoon fails to penetrate

2.5 Tethered Systems and Electrodynamic Tethers:

Japan's JAXA has explored using long conductive tethers to alter debris orbits through electromagnetic drag. The Kounotori Integrated Tether Experiment (KITE) attempted to demonstrate this in 2017, though the tether failed to deploy properly.

Advantages:

- No propellant required for deorbiting

- Passive system once deployed

- Can work on multiple objects

Limitations:

- Very long tethers required (kilometers)

- Deployment complexity

- Limited effectiveness at higher altitudes

- Tether can become debris if it fails

2.6 Laser Ablation and Ion Beam Shepherd:

Concepts for contactless debris removal include using ground-based or space-based lasers to ablate material from debris surfaces, creating thrust, or using ion beams to "push" debris. These remain largely theoretical.

Advantages:

- No physical contact required

- Can engage multiple targets

- Minimal risk to collection vehicle

Limitations:

- Requires enormous power

- Precise targeting over long periods

- Uncertain effectiveness

- Potential for weaponization raises political concerns

2.7 Drag Enhancement Devices:

Drag sails and foam deployment aim to increase atmospheric drag on debris, accelerating natural orbital decay. ESA's ADEO mission tested drag sail deployment.

Advantages:

- Passive deorbiting once deployed

- Low mass and cost

- No propellant needed

Limitations:

-Only effective in LEO (<600 km)

- Slow deorbit process (months to years)

- Requires attachment to debris first

- Ineffective for large, dense objects

2.8 Gap in Current Solutions:

While each of these approaches demonstrates promise, a critical gap remains: no current system combines autonomous operation, multi-target capability, and adaptability to various debris types in a single, cost-effective platform.

Most proposed missions are single-target, requiring dedicated launches for each debris object. This approach is economically unsustainable given the scale of the debris problem. What is needed is a versatile, autonomous spacecraft that can:

- Identify and prioritize multiple debris targets

- Adapt capture methods to debris characteristics

- Operate for extended periods with minimal ground intervention

- Efficiently maneuver between orbital planes

- Safely deorbit collected debris

This research proposes such a system.

—-------------------------------—-----------------------------—-----------------------------—-----------------

3. Methodology

3.1 Research Approach:

This research employs a conceptual design methodology combining:

1. Systems Engineering Analysis - Breaking down the debris removal problem into functional requirements.

2. Orbital Mechanics Calculations - Determining propulsion and maneuvering requirements.

3. Technology Assessment - Evaluating proven and emerging space technologies

4. Parametric Design - Estimating mass, power, and performance characteristics

5. Conceptual Architecture Development - Synthesizing components into an integrated system. The design process follows a top-down approach: defining mission objectives, establishing requirements, proposing solutions, and analyzing feasibility.

3.2 Mission Concept Overview:

Mission Profile

The proposed spacecraft, designated ADCS-1 (Autonomous Debris Collection Spacecraft), operates in Low Earth Orbit (LEO) between 400-800 km altitude. Its mission consists of:

1. Launch and Deployment - Delivered to initial parking orbit

2. Target Identification - Scanning for debris objects using onboard sensors

3. Rendezvous Maneuvers - Approaching selected debris targets

4. Capture Operations - Using hybrid capture system (net + robotic arm)

5. Debris Storage/Attachment - Securing captured objects

6. Deorbit Execution - Controlled atmospheric reentry or graveyard orbit disposal

Design Philosophy

The ADCS-1 concept prioritizes:

- Autonomy: Onboard AI for decision-making to minimize communication delays

- Versatility: Hybrid capture system adapts to different debris types

- Efficiency: Electric propulsion for sustained operations

- Safety: Multiple redundancy and fail-safe mechanisms

- Scalability: Design can be replicated for fleet operations

3.3 Spacecraft Architecture:

The ADCS-1 consists of six major subsystems:

1. Structure and Chassis

- Primary bus: 2m × 2m × 3m

- Modular construction for launch flexibility

- Debris storage compartment capacity: 5-10 objects

2. Propulsion System

- Primary: Hall-effect ion thrusters (4× units)

- Secondary: Chemical RCS thrusters for fine maneuvering

- ΔV budget: 1,500 m/s total mission

- Xenon propellant: 150 kg

3. Power System

- Deployable solar arrays: 8 kW generation

- Lithium-ion batteries: 200 Ah capacity

- Power distribution and management unit

4. Capture System (Hybrid)

Net Deployment Mechanism:

- 4 deployable net launchers

- Net diameter: 6 meters

- Kevlar/Vectran composite material

- Capture range: 5-15 meters

Robotic Manipulator Arm:

- 7-DOF (degrees of freedom) arm

- Reach: 4 meters

- End-effector grip force: 500 N

- Equipped with force/torque sensors

5. Sensors and Navigation

- LIDAR system (range: 100m, resolution: 5cm)

- Optical cameras (wide-angle and telephoto)

- Star trackers for attitude determination

- GPS receiver for orbital positioning

- Inertial measurement unit (IMU)

6. Avionics and Computing

-Radiation-hardened flight computer

- AI/ML processor for autonomous operations

- Communication subsystem (S-band and X-band)

- Data storage: 2 TB solid-state

3.4 Debris Tracking and Target Selection:

The spacecraft employs a three-stage tracking process:

Stage 1: Cataloged Debris Identification

- Pre-mission: Upload orbital elements of known debris from ground catalogs (NORAD, ESA)

- Prioritize targets based on: collision risk, orbital accessibility, size

Stage 2: Onboard Detection

- LIDAR scans within 100m radius

- Optical cameras identify debris characteristics

- AI classifier determines: size, rotation rate, material composition, structural integrity

Stage 3: Approach and Final Tracking

- Continuous LIDAR tracking during rendezvous

- Predictive algorithms calculate debris trajectory

- Real-time collision avoidance

Target Selection Criteria:

1. Debris size: 10 cm to 2 meters (optimal range)

2. Orbital proximity: <50 km altitude difference, <5° inclination difference

3. Rotation rate: <30°/second (for safe capture)

4. Structural integrity: Solid objects preferred over fragile

3.5 Capture Strategy:

The hybrid capture system provides flexibility:

Scenario A: Large, Slow-Rotating Debris (defunct satellites)

- Use robotic arm for controlled grappling

- Arm approaches and clamps onto structural members

- Allows stabilization before attachment

Scenario B: Medium, Fast-Rotating Debris (rocket stages, fragments)

- Deploy net to envelop target

- Net tethers reel in captured debris

- Transfer to storage compartment

Scenario C: Multiple Small Debris

- Sequential net deployments (4 nets available)

- Collect cluster of objects in single operation

- Maximize mission efficiency

3.6 Deorbit Methods:

Depending on debris characteristics and mission phase:

Method 1: Atmospheric Drag Deorbit

-Lower perigee to <300 km

- Natural decay accelerates reentry

- Timeline: 1-6 months depending on altitude

Method 2: Controlled Reentry

- Targeted deorbit burn over ocean

- For large, dense objects

- Minimizes ground casualty risk

Method 3: Graveyard Orbit

- For radioactive or hazardous materials

- Boost to higher, stable orbit (>1000 km)

- Long-term isolation from operational orbits

3.7 Autonomous Operations:

The ADCS-1 employs machine learning algorithms for:

- Debris classification : Trained on simulated debris images

- Trajectory prediction : Neural networks predict tumbling debris motion

- Capture optimization : Reinforcement learning determines optimal approach vectors

- Anomaly detection : Identify system malfunctions autonomously

Ground control involvement is limited to:

- Mission parameter updates

- Target list prioritization

- Emergency override commands.

—-------------------------------—-----------------------------—-----------------------------—-----------------

4.Spacecraft Design

4.1 Overall Configuration:

The ADCS-1 is designed as a modular spacecraft with a central bus and deployable subsystems. Total launch mass is estimated at 1,800 kg, fitting within the payload capacity of medium-lift launch vehicles (Falcon 9, Ariane 6, etc).

Dimensions:

- Stowed: 2m × 2m × 3m

- Deployed (with solar panels): 8m × 2m × 3m

- Robotic arm extended: +4m reach

4.2 Structural Design:

Primary Structure:

- Aluminum alloy honeycomb panels

- Carbon fiber reinforcement for load-bearing members

- Designed to withstand launch loads: 5g acceleration

- Thermal protection: Multi-layer insulation (MLI) blankets

Debris Storage Bay:

- Internal compartment: 1.5m × 1.5m × 2m

- Capacity: Up to 5,000 kg of collected debris

- Securing mechanisms: Restraint straps and magnetic clamps

- Debris mass budget per mission: 3,000 kg average

4.3 Propulsion System Details:

Ion Propulsion (Primary):

- Type: Hall-effect thruster (similar to SpaceX Starlink, OneWeb satellites)

- Thrust per unit: 80 mN

- Specific impulse (Isp): 1,600 seconds

- Power consumption: 1.5 kW per thruster

- Total ΔV capability: 1,200 m/s

Chemical Thrusters (Secondary):

- Type: Hydrazine monopropellant

- 12× 1N thrusters arranged for 6-axis control

- Total hydrazine: 50 kg

- ΔV capability: 300 m/s

- Used for: final approach, emergency maneuvers, attitude control

Propellant Budget:

- Xenon (ion propulsion): 150 kg

- Hydrazine (RCS): 50 kg

- Total propellant mass: 200 kg

4.4 Power System:

Solar Arrays:

- Triple-junction GaAs cells (30% efficiency)

- Deployable panels: 2× wings, each 6m × 2m

- Power generation: 8 kW at beginning of life (BOL)

- Degradation: 0.5% per year

- End-of-life (5 years): 7.8 kW

Energy Storage:

- Lithium-ion battery pack: 200 Ah capacity

- Voltage: 28V nominal

- Eclipse operations: Up to 90 minutes per orbit

- Depth of discharge: 70% maximum

Power Consumption Breakdown:

- Ion thrusters (operational): 6 kW

- Avionics and computing: 500 W

- LIDAR and sensors: 300 W

- Communication: 200 W

- Robotic arm: 400 W (during capture)

- Heating/thermal control: 300 W

- Total peak: 7.7 kW

4.5 Avionics and Computing:

Flight Computer:

- Radiation-hardened processor (RAD750 or equivalent)

- Processing: 200 MIPS

- Memory: 256 GB solid-state storage

- Triple-redundant architecture

AI/ML Processor:

- GPU-based module for neural networks

- Trained models: debris classification, trajectory prediction

- Inference time: <100ms for real-time decisions

Communication:

- S-band: Command uplink (2 kbps) and telemetry downlink (10 kbps)

- X-band: High-speed data downlink (100 Mbps) for imagery

- Ground station passes: 4-6 per day

4.6 Sensors and Navigation System:

LIDAR:

- Type: Scanning laser rangefinder

- Range: 0.5m to 100m

- Angular resolution: 0.1°

- Update rate: 10 Hz

- Point cloud processing for 3D debris mapping

Optical Cameras:

- Wide-angle camera: 90° FOV for situational awareness

- Telephoto camera: 10° FOV for detailed debris inspection

- Resolution: 5 megapixels

- Frame rate: 30 fps

Star Trackers (2× redundant):

- Attitude determination accuracy: 10 arcseconds

- Update rate: 5 Hz

GPS Receiver:

- Orbital position accuracy: <1 meter

- Velocity accuracy: <0.1 m/s

4.7 Capture System Engineering:

Net Deployment System:

Each of 4 net launchers consists of:

- Compressed air ejection mechanism

- Net folded in deployment canister

- 4 weighted projectiles at net corners

- Tether spools with motorized reeling

Net specifications:

- Material: Kevlar-Vectran composite (high tensile strength, low mass)

- Mesh size: 5 cm × 5 cm

- Net mass: 8 kg each

- Tether length: 20 meters

- Tether strength: 5,000 N breaking load

Deployment sequence:

1. Target locked by LIDAR (range: 10-15m)

2. Net ejected at 5 m/s relative velocity

3. Net expands to 6m diameter in 2 seconds

4. Weighted corners wrap around debris

5. Tethers automatically reel in at 0.5 m/s

Robotic Arm:

- Type: 7-degree-of-freedom manipulator

- Joints: 3× shoulder, 1× elbow, 3× wrist

- Reach: 4 meters from mounting point

- Payload capacity: 500 kg at full extension

- End-effector: Adaptive gripper with 3 fingers

Control system:

- Force feedback prevents excessive grip force

- Compliance control absorbs debris rotation energy

- Visual servoing using arm-mounted camera

Grappling fixtures:

- Can interface with standard satellite attachment points

- Magnetic pads for ferrous materials

- Mechanical clamps for structural members

4.8 Mass Budget:

| SUBSYSTEM | MASS (in kgs) |
| --- | --- |
| Structure and Chassis | 400 |
| Propulsion System(dry) | 150 |
| Xenon Propellant | 150 |
| Hydrazine Propellant | 50 |
| Batteries | 80 |
| Avionics and Computing | 60 |
| Sensors(LIDAR, cameras, etc) | 80 |
| Robotic Arm | 200 |
| Net Deployment systems (4x) | 120 |
| Thermal Control | 40 |
| Communication System | 50 |
| Cabling and Harness | 70 |
| Contingency(10%) | 180 |
| Solar Arrays | 100 |
| Total Launch Mass | 1730 kg |
| Debris Collection Capacity | +3000 kg |

4.9 Thermal Control:

Challenges:

- Ion thruster heat: 2 kW dissipation

- Electronics heat: 1 kW dissipation

- Solar exposure: +1,400 W/m² absorbed

- Eclipse cold: -100°C environment

Solutions:

- Radiator panels: 4 m² area

- Heat pipes for thermal distribution

- Multi-layer insulation (MLI)

- Heaters for battery and electronics

- Operational temperature range: -20°C to +50°C

—-------------------------------—-----------------------------—-----------------------------—-----------------

5. Orbital Mechanics Analysis

5.1 Mission Orbit Parameters;

The ADCS-1 operates in a near-circular Low Earth Orbit (LEO):

Initial Parking Orbit:

- Altitude: 600 km

- Inclination: 98° (sun-synchronous)

- Eccentricity: ~0 (circular)

- Period: ~96.7 minutes

Rationale:

- High debris concentration in 600-800 km range

- Sun-synchronous orbit provides consistent solar illumination

- Inclination allows access to most LEO debris

5.2 Fundamental Orbital Equations:

Orbital Velocity:

For a circular orbit:

v = √(μ/r)

Where:

- μ = GM_Earth = 398,600 km³/s² (Earth's gravitational parameter)

- r = R_Earth + h = 6,371 km + 600 km = 6,971 km

Calculating:

v = √(398,600/6,971) = 7.56 km/s

Orbital Period:

T = 2π√(r³/μ)

T = 2π√(6,971³/398,600) = 5,802 seconds ≈ 96.7 minutes

Orbital Energy:

The specific orbital energy:

ε = -μ/(2a)

Where a is the semi-major axis. For our 600 km circular orbit:

ε = -398,600/(2 × 6,971) = -28.6 MJ/kg

5.3 Delta-V (ΔV) Budget Analysis:

The ΔV budget determines the total velocity change capability needed for all mission

maneuvers.

1. Orbit Raising/Lowering:

To change altitude between debris targets, we use the Hohmann transfer approximation:

ΔV_transfer = |v₁ - v₂|

For altitude change from 600 km to 700 km:

v₁ = √(398,600/6,971) = 7.56 km/s

v₂ = √(398,600/7,071) = 7.51 km/s

ΔV ≈ 50 m/s (per 100 km altitude change)

2. Plane Change Maneuvers:

Inclination changes are expensive:

ΔV_plane = 2v sin(Δi/2)

For a 5° inclination change at 600 km:

ΔV = 2(7.56) sin(5°/2) = 0.66 km/s = 660 m/s

Strategy: Minimize plane changes by targeting debris in similar orbital planes.

3. Rendezvous and Approach:

Fine maneuvering near debris:

ΔV_approach ≈ 20-50 m/s per target

Includes:

- Phase adjustment: 10-20 m/s

- Final closure: 5-10 m/s

- Station-keeping: 5-10 m/s

- Separation after capture: 10 m/s

4. Deorbit Maneuver:

To lower perigee for atmospheric reentry:

ΔV_deorbit = v - v_new For deorbit from 600 km to 300 km perigee:

ΔV ≈ 150 m/s

Total Mission ΔV Budget:

| Maneuver Type | ∆V (m/s) | Number of Operations |
| --- | --- | --- |
| Altitude Adjustments | 50 | 10 targets |
| Plane Changes | 200 | Small corrections only |
| Rendezvous Approaches | 30 | 10 targets |
| Station-keeping | 100 | Continuous |
| Deorbit Maneuver | 150 | 1 |
| Contingency(20%) | 240 | Reserve |
| TOTAL | 1490 m/s |  |

This is within our spacecraft's 1,500 m/s capability (1,200 from ion + 300 from chemical).

5.4 Hohmann Transfer Calculation Example:

Scenario: Transfer from 600 km to 700 km altitude

Step 1: Calculate initial and final orbit radii:

- r₁ = 6,971 km

- r₂ = 7,071 km

Step 2: Calculate transfer orbit semi-major axis:

- a_transfer = (r₁ + r₂)/2 = 7,021 km

Step 3:  First burn (raise apogee):

ΔV₁ = √(μ/r₁) × [√(2r₂/(r₁+r₂)) - 1]

ΔV₁ = 7.56 × [√(2×7,071/14,042) - 1]

ΔV₁ = 7.56 × [1.0036 - 1] = 27 m/s

Step 4: Second burn (circularize at apogee):

ΔV₂ = √(μ/r₂) × [1 - √(2r₁/(r₁+r₂))]

ΔV₂ = 7.51 × [1 - √(2×6,971/14,042)]

ΔV₂ = 7.51 × [1 - 0.9964] = 27 m/s

Total ΔV: 27 + 27 = 54 m/s

Transfer time: Half orbital period at transfer orbit ≈ 48 minutes

5.5 Rendezvous Strategy:

Phase 1: Far-range approach (100 km to 10 km)

- Use GPS and catalog data for navigation

- Perform phasing maneuvers to synchronize orbits

- Time: 1-2 orbits (~2-3 hours)

- ΔV: 10-15 m/s

Phase 2: Close-range approach (10 km to 100 m)

-LIDAR becomes primary sensor

- Continuous trajectory updates

- Relative velocity: <1 m/s

- Time: 30-60 minutes

- ΔV: 5-10 m/s

Phase 3: Final approach (100 m to capture)

- Optical cameras and LIDAR tracking

- Match debris rotation if possible

- Approach velocity: 0.1-0.5 m/s

- Time: 10-20 minutes

- ΔV: 5 m/s

5.6 Orbital Decay and Atmospheric Drag:

Atmospheric Drag Force:

F_drag = ½ ρ v² C_D A

Where:

- ρ = atmospheric density (varies with altitude)

- v = orbital velocity

- C_D = drag coefficient (~2.2 for spacecraft)

- A = cross-sectional area (~8 m² for ADCS-1)

At 600 km altitude:

- ρ ≈ 1.5 × 10⁻¹³ kg/m³

- v = 7,560 m/s

- F_drag = ½(1.5×10⁻¹³)(7,560²)(2.2)(8) ≈ 7.5 × 10⁻⁴ N

Orbital Decay Rate:

The rate of altitude loss:

dh/dt = -F_drag × v / (m × g)

For ADCS-1 (mass 1,800 kg):

dh/dt ≈ -3 m/day at 600 km

Natural Decay Timeline:

- 600 km: ~10-15 years to reentry

- 500 km: ~2-3 years

- 400 km: ~6 months

- 300 km: ~1 month

5.7 Propellant Consumption:

Ion Propulsion:

Tsiolkovsky rocket equation:

ΔV = I_sp × g₀ × ln(m₀/m_f)

Where:

- I_sp = 1,600 seconds (specific impulse)

- g₀ = 9.81 m/s²

- m₀ = initial mass

- m_f = final mass

For ΔV = 1,200 m/s:

1,200 = 1,600 × 9.81 × ln(1,800/m_f)

m_f = 1,800/e^(0.0764) = 1,667 kg

Propellant mass: 1,800 - 1,667 = 133 kg ✓ (within 150 kg xenon budget)

5.8 Mission Timeline Example:

Sample 30-day mission collecting 5 debris objects:

| Day | Activity | ∆V(m/s) |
| --- | --- | --- |
| 0-1 | Launch and checkout | 0 |
| 2-4 | Transfer to debris #1 | 60 |
| 5 | Capture #1 | 30 |
| 6-8 | Transfer to debris #2 | 40 |
| 9 | Capture #2 | 30 |
| 10-13 | Transfer to debris #3 | 70 |
| 14 | Capture #3 | 30 |
| 15-18 | Transfer to debris #4 | 80 |
| 19 | Capture #4 | 30 |
| 20-24 | Transfer to debris #5 | 50 |
| 25 | Capture #5 | 30 |
| 26-30 | Deorbit preparation and burn | 150 |
| TOTAL |  | 600 m/s |

*Remaining ΔV for extended mission: 900 m/s

5.9 Collision Avoidance:

The probability of collision with tracked debris during operations:

P_collision = n × σ × v × t

Where:

- n = debris number density

- σ = collision cross-section

-v = relative velocity

- t = exposure time

Risk mitigation:

- Continuous tracking of all cataloged objects within 50 km

- Automated avoidance maneuvers for close approaches (<1 km)

- ΔV reserve: 100 m/s for emergency maneuvers

—-------------------------------—-----------------------------—-----------------------------—-----------------

6. Results & Discussion

6.1 Design Performance Summary:

The conceptual ADCS-1 spacecraft demonstrates the following capabilities:

Mission Capacity:

- Debris targets per mission: 5-10 objects

- Total debris mass collection: Up to 3,000 kg

- Operational altitude range: 400-800 km

- Mission duration: 30-90 days (configurable)

- ΔV margin: 1,500 m/s total

Capture System Performance:

- Net capture success (estimated): 85% for tumbling debris

- Robotic arm success (estimated): 95% for large, slow-rotating objects

- Capture range: 5-15 meters (net), 0-4 meters (arm)

- Target size range: 10 cm to 2 meters optimal

Efficiency Metrics:

- ΔV per debris object: 60-150 m/s average

- Cost per debris removal: Estimated $2-5 million (amortized)

- Propellant efficiency: Ion propulsion provides 5× better Isp than chemical

6.2 Comparison to Existing Concepts:

| Feature | ADCS-1(this mission) | ClearSpace-1 | RemoveDEBRIS |
| --- | --- | --- | --- |
| Targets per mission | 5-10 | 1 | Demonstration only |
| Capture method | Hybrid(net + arm) | Robotic Arm | Net & harpoon |
| Autonomy | High (AI-based) | Semi-Autonomous | Ground-controlled |
| Propulsion | Electric + Chemical | Chemical | None (hosted payload) |
| Mission duration | 30-90 Days | Single target | Limited demo |
| Reusability | Multi target capable | Single use | N/A |

6.3 Economic Analysis:

Development Costs (Estimated):

- Spacecraft design and engineering: $50-80 million

- Prototype construction: $100-150 million

- Testing and qualification: $30-50 million

- Launch services: $30-60 million (rideshare options)

- Total first mission: ~$210-340 million

Operational Costs Per Mission:

- Ground operations: $5-10 million

- Propellant and consumables: $2 million

- Data processing and analysis: $1 million

- Total per mission: ~$8-13 million

Cost Per Debris Object:

- First mission (10 targets): $22-35 million each

- Subsequent missions (fleet of 5 spacecraft): $3-5 million each

Comparison: Current single-target missions estimate $100-300 million per debris removal. ADCS-1 offers 10-30× cost reduction at scale.

6.4 Environmental Impact:

Debris Removal Rate:

- Single ADCS-1: 50-100 objects per year

- Fleet of 10 spacecraft: 500-1,000 objects per year

- Current debris growth: ~200-300 new trackable objects per year

Sustainability Assessment:

A fleet of 10-15 ADCS-1 spacecraft could:

- Stabilize debris population in critical orbits (600-800 km)

- Remove high-risk collision threats

- Enable sustainable long-term space operations

Kessler Syndrome Mitigation: By targeting the most collision-prone objects, the mission directly addresses the cascade

collision risk.

6.5 Technical Feasibility Assessment:

Technology Readiness Levels (TRL):

| Subsystem | TRL | Status |
| --- | --- | --- |
| Ionic Propulsion | 9 | Flight-proven (starlink, Dawn Mission ) |
| Robotic Arm | 8-9 | ISS Candarm heritage |
| Net Capture | 7 | RemoveDEBRIS demonstration |
| LIDAR tracking | 9 | Commercial spacecraft use |
| Autonomous AI | 6-7 | Requires space qualification |
| Integrated System | 4-5 | Conceptual, needs development |

Critical Path Items:

1. Integration and testing of hybrid capture system

2. AI algorithm validation in space environment

3. Radiation hardening of AI processors

4. Multi-target mission operations procedures

Development Timeline (Estimated):

- Year 1-2: Detailed design and ground prototyping

- Year 3-4: Component testing and qualification

- Year 5: Integration and system testing

- Year 6: Launch and commissioning

- Year 7+: Operational missions

6.6 Operational Challenges:

1. Debris Characterization:

- Unknown material composition affects capture strategy

- Rotation rates may exceed safe capture limits (>30°/s)

- Structural integrity uncertain (may fragment on contact)

Solution: Conservative approach with extensive pre-capture assessment.

2. Communication Delays:

- Ground control commands take seconds to reach spacecraft

- Autonomous decisions required for time-critical operations

Solution: High-level mission commands with onboard autonomy.

3. Propellant Management:

- ΔV budget must be carefully managed

- Uncertainty in debris positions requires margin

Solution: 20% ΔV contingency reserve.

4. Space Weather:

- Solar activity affects atmospheric density

- Geomagnetic storms impact drag predictions

Solution: Real-time atmosphere models and adaptive planning.

6.7 Safety Considerations:

Collision Risk During Capture:

- Approaching tumbling debris is high-risk

- Net deployment may destabilize debris

Mitigation:

- Abort criteria programmed into flight software

- Multiple trajectory prediction scenarios

- Escape maneuvers pre-calculated

Secondary Debris Creation:

- Capture attempts may fragment brittle objects

- Net tethers could break and create new debris

Mitigation:

-Structural assessment before capture attempts

- Over-engineered tether strength (5× safety factor)

- Contingency procedures for failed captures

Reentry Safety:

- Controlled reentry over oceans

- Debris survivability analysis for ground impact risk.

Mitigation:

- Reentry corridors avoid populated areas

- Tracking during reentry phase

—-------------------------------—-----------------------------—-----------------------------—-----------------

7. Limitations & Challenges

7.1 Technical Limitations:

1. Target Size Constraints

- Optimal debris size: 10 cm to 2 meters

- Too small (<10 cm): Difficult to detect and track with onboard sensors

- Too large (>2 meters):Exceeds capture system capacity and mass limits

- Implication: Cannot address entire debris spectrum

2. Orbital Accessibility

- Plane changes are ΔV-expensive (660 m/s per 5° inclination)

- Limited to debris within ±5° inclination of spacecraft orbit

- Cannot efficiently service polar and equatorial orbits in single mission

-  Implication: Multiple spacecraft with different inclinations needed for comprehensive

coverage

3. Capture System Limitations

- Net system: 4 nets total (single-use), limits targets per mission

- Robotic arm: Cannot safely capture rapidly tumbling debris (>30°/s)

- Fragile debris may break during capture

- Implication: Mission success depends on debris structural integrity

4. Power Constraints

- Ion thrusters require high power (6 kW)

- Eclipse periods limit thruster operation time

- Solar array degradation over 5-year lifespan

- Implication: Mission duration limited by power availability

5. Communication Bandwidth

- Limited ground station passes (4-6 per day)

- High-resolution imagery requires X-band downlink

- Autonomous decisions made without real-time ground input

- Implication: Trust in AI algorithms critical

7.2 Operational Challenges:

1. Debris Tracking Uncertainty

- Catalog orbital elements have uncertainty (±100m-1km)

-Small debris not cataloged by ground systems

- Solar radiation pressure affects trajectory predictions

- Challenge: Requires onboard sensors to compensate for uncertainty

2. Unpredictable Debris Behavior

- Unknown rotation rates until close approach

- Internal structure unknown (solid vs hollow)

- Material composition affects capture strategy

- Challenge: Cannot fully plan capture until final approach

3. Space Weather Variability

- Solar flares increase atmospheric density

- Drag predictions become inaccurate

- Orbital decay rates change unexpectedly

- Challenge: Requires adaptive mission planning

4. Multi-debris Logistics

- Optimal debris sequencing is complex optimization problem

- Propellant constraints limit accessible targets

- Time windows for capture opportunities limited

- Challenge: AI-based planning algorithms under development

7.3 Economic and Policy Limitations:

1. High Development Costs

- Estimated $200-300 million for first mission

- Return on investment requires multiple successful missions

- Funding challenges in public and private sectors

- Limitation: Requires sustained financial commitment

2. Regulatory Uncertainty

- International space law unclear on debris removal rights

- Liability for secondary debris creation

- Permission needed to approach foreign satellites

- Limitation: Legal framework needs development alongside technology

3. Business Model Challenges

- No established market for debris removal services

- Who pays for removing abandoned satellites?

- Tragedy of the commons: debris affects everyone, benefits all

- Limitation: Requires government support or innovative financing

7.4 Safety Concerns:

1. Collision Risk

- Operating near debris increases collision probability

- Failed capture may create secondary debris

- Net or tether failure adds new hazards

- Concern: Mission could worsen debris problem if not executed carefully

2. Active Satellite Interference

- ADCS-1 operates in crowded LEO environment

- Risk of unintended approach to operational satellites

-Communication interference possible

- Concern: Requires strict operational protocols

3. Reentry Hazards

- Large debris may survive reentry

- Ground impact risk, although low probability

- Hazardous materials (batteries, propellant) may survive

- Concern: Controlled reentry procedures essential

7.5 Technology Gaps:

1. AI Algorithm Validation

- Machine learning models trained on simulated data

- Space environment may introduce unforeseen scenarios

- Edge cases difficult to predict and test

- Gap: Extensive in-orbit validation needed

2. Long-Duration Autonomous Operations

- Current satellites rely heavily on ground control

- ADCS-1 requires weeks of autonomous operation

- Fault detection and recovery must be robust

- Gap: Heritage in autonomous spacecraft limited

3. Hybrid Capture System Integration

- Net and robotic arm combination not previously flown

- Coordination between systems requires novel software

- Simultaneous operation scenarios untested

- **Gap:** Ground testing cannot fully replicate space conditions

7.6 Simulation and Modeling Limitations:

Current Status:

- Orbital mechanics calculated using idealized equations

- Capture dynamics modeled analytically, not simulated

- Debris structural response to capture unknown

- AI algorithms trained but not validated in realistic scenarios

Required Future Simulations:

1. High-fidelity orbital dynamics: Multi-body gravitational effects, atmospheric drag

variations

2. Capture mechanism simulations:*Finite element analysis of net deployment, robotic arm

contact dynamics

3. Hardware-in-the-loop testing: Real sensors and computers tested with simulated

environment

4. Monte Carlo mission simulations: Thousands of mission scenarios to assess success

probability

Implication: This research provides conceptual foundation, but extensive simulation work

remains before hardware development.

7.7 Scalability Considerations

Fleet Operations:

-Coordination between multiple ADCS-1 spacecraft

- Avoiding redundant targeting of same debris

- Ground operations complexity increases with fleet size

- Challenge: Centralized or distributed mission planning?

Debris Population Growth:

- New debris created faster than removal rate (currently)

- Requires 50-100 spacecraft to stabilize LEO debris population

- Economic feasibility of large-scale deployment uncertain

- Challenge: Need complementary prevention measures (better spacecraft design, end-of-life compliance)

—-------------------------------—-----------------------------—-----------------------------—-----------------

8. Future Work

The conceptual design presented in this research establishes a foundation for an autonomous debris collection spacecraft. However, significant work remains to transform this concept into operational reality. This section outlines the critical next steps.

8.1 Simulation and Modeling (Immediate Priority):

1. Orbital Dynamics Simulation

- Software:  STK (Systems Tool Kit), GMAT (General Mission Analysis Tool), or custom Python (poliastro, Orekit)

- Objectives:

- Simulate multi-target rendezvous sequences

- Optimize ΔV expenditure for mission efficiency

- Model atmospheric drag variations with space weather

- Validate Hohmann transfer calculations with perturbations

- Timeline: 3-6 months

2. Capture Mechanism Physics Simulation

- Approach: Finite Element Analysis (FEA) and multibody dynamics

- Objectives:

- Model net deployment in microgravity

- Simulate tether dynamics during debris capture

- Analyze robotic arm contact forces

- Predict debris structural response to capture

- Software: ANSYS, Adams, or Gazebo (robotics simulation)

- Timeline: 6-12 months

3. AI Algorithm Development and Testing

- Focus Areas:

- Debris classification from LIDAR/camera data

- Trajectory prediction for tumbling objects

- Reinforcement learning for optimal approach paths

- Anomaly detection for system health monitoring

- Training Data: Simulated debris images, orbital dynamics datasets

- Validation: Hardware-in-the-loop testing with real sensors

- Timeline: 12-18 months

4. Mission Monte Carlo Analysis

-Purpose: Assess mission success probability under uncertainty

- Variables: Debris position uncertainty, capture success rates, propellant consumption variability

- Outcome: Risk assessment and contingency planning

- Timeline: 3-6 months

8.2 Hardware Prototyping and Testing:

1. Net Deployment System Ground Testing

- Build scaled prototype net launcher

- Test deployment dynamics in parabolic flight (microgravity simulation)

- Measure deployment velocities and net expansion

- Validate tether reeling mechanisms

- Timeline: 12-18 months

- Cost: $2-5 million

2. Robotic Arm Prototype

- Develop 7-DOF arm testbed

- Test end-effector grappling with mock debris

- Validate force-feedback control algorithms

- Compliance testing for energy absorption

- Timeline: 18-24 months

- Cost: $5-10 million

3. Sensor Suite Integration

- Integrate LIDAR, cameras, and IMU on test platform

- Develop sensor fusion algorithms

- Test in representative environments (thermal-vacuum chamber)

- Validate debris tracking at various ranges

- Timeline: 12-18 months

- Cost: $3-6 million

4. Propulsion System Testing

- Procure or develop Hall-effect thruster

- Vacuum chamber testing for thrust and Isp validation

- Thermal characterization

- Long-duration endurance testing (1,000+ hours)

- Timeline: 18-24 months

- Cost: $10-15 million

8.3 Systems Integration and Mission Design:

1. Spacecraft Conceptual Design Refinement

- Detailed CAD modeling of all subsystems

- Mass properties analysis (center of gravity, moments of inertia)

- Thermal analysis with realistic orbital heating/cooling

- Structural analysis for launch loads

- Software: SolidWorks, CATIA, Thermal Desktop

- Timeline: 6-12 months

2. Power System Sizing

- Solar array layout optimization

- Battery charge/discharge cycle analysis

-  budget validation with realistic operation profiles

- Contingency power management

- Timeline: 3-6 months

3. Communication Architecture

- Ground station network design

- Data link budget analysis

- Command and telemetry protocol development

- Autonomous operations concept of operations (ConOps)

- Timeline: 6-9 months

4. Avionics and Software Architecture

- Flight computer selection and redundancy design

- Software architecture for autonomous operations

- Fault detection, isolation, and recovery (FDIR) algorithms

- Ground software for mission planning and monitoring

- Timeline: 12-24 months

8.4 Partnerships and Collaboration:

1. Academic Collaboration

- Partner with aerospace engineering departments for research

- Graduate student projects on specific subsystems

- Access to university facilities (wind tunnels, vacuum chambers, clean rooms)

- Potential Partners: MIT, Caltech, Stanford, Georgia Tech, Purdue, University of Colorado

2. Industry Partnerships

- Collaborate with spacecraft manufacturers for design reviews

- Propulsion vendors for ion thruster integration

- Sensor manufacturers for space-qualified hardware

- Potential Partners: Northrop Grumman, Lockheed Martin, Busek (propulsion), SSL/Maxar

3. Space Agency Support

- Seek NASA SBIR/STTR funding for technology development

- ESA Clean Space initiative collaboration

- JAXA debris removal program coordination

- Funding Opportunities: NASA SBIR Phase I ($150k), Phase II ($1M+)

4. International Cooperation

- Share debris tracking data with space agencies

- Coordinate mission planning to avoid operational satellites

- Contribute to international debris mitigation standards

- Organizations: Inter-Agency Space Debris Coordination Committee (IADC), COPUOS

8.5 Demonstration Mission Planning:

Phase 1: Component Testing in Orbit

- Deploy net or robotic arm as secondary payload on ISS or commercial satellite

- Demonstrate capture mechanism in space environment

- Validate sensor performance

- Timeline: 3-5 years from project start

- Cost: $20-40 million

Phase 2: Single-Target Demonstration Mission

- Small spacecraft targeting one known debris object

- Prove end-to-end operation: rendezvous, capture, deorbit

- Validate autonomous AI algorithms

- Timeline: 5-7 years from project start

- Cost: $100-150 million

Phase 3: Full-Scale Multi-Target Mission

- ADCS-1 as designed in this research

- Capture 5-10 debris objects

- Demonstrate economic viability

- Timeline: 7-10 years from project start

- Cost: $200-300 million

8.6 Policy and Legal Development:

1. Regulatory Framework

- Work with national space agencies on debris removal permissions

- Develop international guidelines for active debris removal

- Liability agreements for secondary debris creation

-Timeline: Ongoing, parallel to technical development

2. Business Model Development

- Identify potential customers (satellite operators, governments)

- Develop pricing models for debris removal services

- Explore public-private partnerships

- Consider insurance industry involvement

- Timeline: 2-4 years

3. Public Outreach and Education

- Communicate importance of space sustainability

- Engage public in supporting debris removal missions

- STEM education initiatives around space debris

- Timeline: Ongoing

8.7 Technology Advancement Roadmap:

Near-Term (1-3 years):

- Complete high-fidelity simulations

- Build and test ground prototypes

- Develop AI algorithms

- Secure initial funding

Mid-Term (3-7 years):

- In-orbit demonstration of capture mechanism

- Partnerships with launch providers

- Single-target demonstration mission

- Refine design based on flight data

Long-Term (7-15 years):

- Operational fleet of ADCS-1 spacecraft

- Expand to medium Earth orbit (MEO) debris

- Develop next-generation autonomous systems Achieve economically sustainable debris removal

8.8 Research Questions for Further Investigation:

1. Optimal mission sequencing: What algorithm minimizes ΔV for multi-target missions with

orbital constraints?

2. Debris structural assessment: Can LIDAR or radar determine debris fragility before

capture?

3. Swarm coordination: How can multiple ADCS-1 spacecraft cooperate to maximize debris

removal?

4. Capture mechanism innovation: Are there better alternatives to nets and robotic arms?

5. Propulsion alternatives: Could nuclear electric propulsion extend mission capabilities?

6. Debris recycling: Can collected debris be processed in orbit for materials?

8.9 Success Metrics:

To evaluate progress toward operational capability:

Technical Metrics:

- Simulation accuracy: <5% error in ΔV predictions

- Capture success rate: >80% in simulations

- AI classification accuracy: >95%

- Hardware reliability: Mean time between failures >10,000 hours

Programmatic Metrics:

- Secure $50M+ in development funding by Year 2

- Demonstrate capture mechanism in orbit by Year 5

- Launch full mission by Year 8

- Achieve cost of <$10M per debris removal by Year 10

Impact Metrics:

- Remove 50+ debris objects by Year 10

- Reduce collision risk in LEO by measurable percentage

- Establish international standards for debris removal

- Enable sustainable growth of space economy

—-------------------------------—-----------------------------—-----------------------------—-----------------

9. Conclusion

Space debris represents one of the most significant threats to the future of space exploration and the growing space economy. With millions of objects now orbiting Earth at destructive velocities, the risk of catastrophic collisions grows daily. Without intervention, the Kessler Syndrome could render valuable orbital regions unusable for generations. This research has presented a conceptual design for the **ADCS-1 (Autonomous Debris Collection Spacecraft)**, an innovative solution to the space debris crisis. The key contributions of this work include:

9.1 Principal Achievements:

1. Comprehensive Spacecraft Design

A detailed conceptual architecture integrating:

-Hybrid capture system (net deployment + robotic arm) providing versatility

- Electric propulsion (Hall-effect thrusters) enabling extended missions

- Autonomous AI-based navigation reducing operational costs

- Multi-target capability achieving 5-10 debris removals per mission

- Total mass budget of 1,730 kg, compatible with commercial launch vehicles

2. Rigorous Orbital Mechanics Analysis

Mathematical framework demonstrating:

- Total ΔV budget of 1,500 m/s sufficient for multi-target missions

- Hohmann transfer calculations for altitude changes

- Propellant consumption estimates confirming feasibility

- Mission timeline showing 30-90 day operational duration

- Cost reduction to $3-5 million per debris object (vs. $100-300M for single-target missions)

3. Technology Integration

Synthesis of proven space systems:

- TRL 7-9 technologies for most subsystems

- Net capture demonstrated by RemoveDEBRIS

- Robotic manipulation heritage from ISS operations

- Ion propulsion validated on commercial satellites

- Integration challenges identified for development planning

4. Honest Assessment of Limitations

Transparent identification of:

- Orbital plane change constraints

- Debris size limitations (10 cm to 2 meters optimal)

- Capture success dependencies on debris characteristics

- Regulatory and economic hurdles

- Technology gaps requiring further development

9.2 Innovation and Impact:

The ADCS-1 concept represents a paradigm shift in debris removal strategy:

Economic Viability: Multi-target capability reduces per-debris cost by 10-30× compared to single-target missions, making large-scale debris removal economically feasible for the first time.

Operational Autonomy: AI-driven decision-making enables weeks of autonomous operation,

dramatically reducing ground operations costs and enabling rapid response to emerging threats.

Adaptive Capture: Hybrid net-and-arm system provides flexibility to handle diverse debris

types, from tumbling rocket stages to defunct satellites, maximizing mission success rates.

Scalability: Modular design allows fleet operations, with 10-15 spacecraft capable of

stabilizing debris population growth in critical LEO zones.

9.3 Feasibility Assessment:

Technical Feasibility: HIGH

All major subsystems rely on proven or demonstrated technologies. The primary technical

The challenge lies in integration and autonomous coordination rather than fundamental technology gaps.

Economic Feasibility: MODERATE

Development costs ($200-300M) are comparable to commercial satellite programs. Operational economics become favorable with fleet deployment, though business model development remains critical.

Political Feasibility: MODERATE

International support for debris mitigation is growing, but regulatory frameworks lag behind

technical capabilities. Success requires parallel development of legal and technical

solutions.

9.4 Broader Implications:

Beyond debris removal, this research demonstrates:

Autonomous Space Operations: AI-driven spacecraft capable of complex decision-making

represent the future of space missions, from asteroid mining to planetary exploration.

Sustainability Paradigm: Active debris removal establishes precedent for environmental

stewardship in space, analogous to ocean cleanup or atmospheric pollution control on Earth.

International Cooperation: Debris affects all spacefaring nations equally, creating

opportunities for collaborative missions and shared infrastructure.

Economic Opportunities: A debris removal industry could generate thousands of jobs and

billions in economic activity while ensuring long-term access to space.

9.5 Path Forward:

While this research provides a strong conceptual foundation, the path to operational

capability is clear:

Immediate (1-3 years): Complete high-fidelity simulations, build ground prototypes,

develop and validate AI algorithms, secure development funding.

Near-term (3-7 years): In-orbit demonstration of capture mechanisms, single-target proof-

of-concept mission, partnerships with space agencies.

Long-term (7-15 years): Operational fleet deployment, establishment of sustainable debris

removal services, international coordination frameworks.

9.6 Final Thoughts:

The space debris problem is solvable, but time is critical. Each year of inaction sees hundreds of new debris objects created, raising collision risks exponentially. The ADCS-1 concept presented here offers a technically feasible, economically viable path forward. This research demonstrates that debris removal does not require revolutionary breakthrough technologies—what is needed is integration of existing systems, sustained investment, and international will. The technology exists today to begin reversing decades of orbital pollution.

Future generations will depend on our orbital environment remaining accessible. Whether for communications, navigation, Earth observation, scientific research, or human exploration,space is now integral to modern civilization. Preserving this environment is not optional—it is imperative.  The ADCS-1 represents one possible solution among many that will be needed. But it exemplifies the engineering mindset required: practical, scalable, economically sustainable, and technically achievable with current capabilities.

Space debris removal is not a question of if we can do it, but whether we will commit to doing it before the problem becomes irreversible.

This research takes the first step on that journey.

10. References

De

—

Suggested Further Reading

For readers interested in deeper exploration of topics covered in this research:

- Orbital Mechanics: Battin, R. H. (1999). *An Introduction to the Mathematics and Methods of Astrodynamics*. AIAA.

- Space Systems Engineering: Fortescue, P., et al. (2011). *Spacecraft Systems

Engineering* (4th ed.). Wiley.

- Debris Modeling: Klinkrad, H. (2006). *Space Debris: Models and Risk Analysis*.

Springer-Praxis.

- Autonomous Systems: Thrun, S., Burgard, W., & Fox, D. (2005). *Probabilistic Robotics*. MIT Press.

Acknowledgments

This conceptual research was developed as an independent study to contribute to the growing body of knowledge on active debris removal and sustainable space operations. The author acknowledges the wealth of publicly available information from space agencies (NASA, ESA, JAXA), academic institutions, and industry partners that made this research possible. Special recognition to the researchers and engineers working on RemoveDEBRIS, ClearSpace-1, and other pioneering debris removal projects whose demonstrations provide critical validation of the technologies proposed in this concept.

---

Document Information:

- Title: A Conceptual Design for an Autonomous Orbital Vehicle for Space Debris

Collection and Decommissioning of Defunct Satellites

- Author: Adithya Boyini

- Date Started : September 2, 2025

- **Status:** Conceptual Design - Simulation and Prototyping Phase Pending.